## PCM SYSTEM FOR VOICE TRANSMISSION  FOR REAL TIME CONSTRUCTION

REAL TIME PROJECT REPORT

Submitted by

| V.NAVYA | (Reg No.23891A0458) |
| --- | --- |
| S.SAI DATTU | (Reg No.23891A0451) |
| T.VISHAL REDDY | (Reg No. 23891A0456) |
| V.ESHWAR | (Reg NO.23891A0462) |

Under the guidance of

### Mr. J.Ramesh

Assistant Professor

in partial fulfillment of the degree of

BACHELOR OF TECHNOLOGY

in

### ELECTRONICS AND COMMUNICATION ENGINEERING

MAY 2025

CERTIFICATE

This	is	to	our	certify	that	project	work	is	titled	about	the	technique

“PCM SYSTEM FOR VOICE TRANSMISSION   FOR REAL TIME CONSTRUCTION” submitted by  V.NAVYA (Reg No.23891A0458)  S.SAI DATTU (Reg No 23891A0451)  T.VISHAL REDDY(Reg NO.23891A0456)  V.ESHWAR (Reg NO 23891A0462), in partial fulfillment of the requirements for the award of the degree of Bachelor of Technology in electronics and communication enginnering to the Vignan Institute of Technology And Science, Deshmukhi is a record of  bonafide work carried out by them under my guidance and supervision.

The results embodied in this project report have not been a submitted in any too university for the award of any degree, and the results are achieved satisfactorily.

| Mr. J. RAMESH |  |
| --- | --- |
| Project Guide | Head of the Department |

External Examiner

## DECLARATION

We hereby declare that the entiled is bonafide work duly completed by us. It does not contain any part of the project submitted by any other candidate to this or any other institute of the university. All such materials that have been obtained from other sources have been duly acknowledged

## ACKNOWLEDGEMENT

The fulfillment that accompanies the successful completion of our project, this is our moment to convey our heartfelt gratitude to all those who have played an instrumental role in our mini project’s realization. Sincere thanks are extended to Dr. L. Rathaiah, the esteemed Chairman of Vignan Group of Institutions.

Our expression of gratitude further extends to Mr. Boyapati Shravan, the distinguished CEO of Vignan Group of Institutions, Hyderabad.

We also extend our heartfelt gratitude to Dr. G. Durga Sukumar, our esteemed Principal, for generously providing us with the necessary facilities and resources that were instrumental in the successful undertaking and completion of this minor project.

Our profound gratitude goes to,DR.P.A Harsha vardhini, Professor and Head of the Department, whose unwavering encouragement and support have been a source of inspiration throughout the course of our project. Her guidance and mentorship have played a pivotal role in shaping our project and fostering a conducive learning environment. We are deeply indebted to her for her invaluable contributions and for being a guiding light on our academic journey.

We are also very grateful to project coordinator, MR.J. RAMESH Assistant professor, for her help in our project solving problems during project.

Our sincere thanks to our guide MR.J. RAMESH Assistant Professor for her guidance, constant supervision, and necessary information regarding the mini project, as well as her support in completing it.

Last but not least, we wish to express our gratitude and thanks to friends and beloved parents for their support and help.

# ABSTRACT

Effective communication is vital on construction sites to ensure coordination, safety, and operational efficiency. Traditional analog systems often suffer from noise interference, limited range, and signal degradation, which can compromise performance in challenging site environments. To address these issues, this project proposes a real-time voice communication system using Pulse Code Modulation (PCM), a robust digital encoding technique.

PCM works by converting analog voice signals into digital data through three primary steps: sampling, quantization, and encoding. This digital approach significantly improves signal clarity, reduces susceptibility to environmental noise, and enables reliable long-distance transmission when integrated with wireless technologies such as RF or Bluetooth.

The system comprises microphones, ADCs, microcontrollers for PCM encoding, wireless transmission modules, and DACs at the receiver end to reproduce the original voice. It ensures low-latency and high-fidelity communication, even in noisy and dynamic environments typical of construction sites.

Key benefits include improved audio quality, reduced distortion, enhanced safety through clearer alerts, and easier integration with modern digital infrastructure. The PCM system can support both worker-to-worker and worker-to-control room communication, making it an ideal solution for real-time construction site applications.

Future advancements may include integration with IoT, AI-based voice analysis, and automated alerts to further enhance site management and safety protocols.

### TABLE OF CONTENTS

ABSTRACT	v

LIST OF FIGURES	viii

#### LIST OFABBREVIATIONS.....................................………..……...xi

- INTRODUCTION.......................................................................................... 1

1.1 objectives of the project…………………………………………………1

1.2 scope of the project……………………………………………………..1

  - Significance of the PCM  in voice transmission…………………............3

  - Over view of voice transmission system…….............................................4

- LITERATURE SURVEY………..........................................................…....8

2.1.Historical background of PCM…………………. .........................…….8

2.2 Existing Voice Transmission technologies……………………................8

2.3.Comparision between analog and digital methods……...........................10

2.4.Gaps in existing system ..........................................................................11

3. SYSTEM DESIGN…………………………………………………………..12

3.1 PCM system Architecture…………………...........................................….. 12

3.2 System block diagram………………………….................................…12

3.3. functional components…….................................................................…....13

3.4.Data flow and signal path…………… ..................................................….. 13

3.5.Hardware and software requirements.......................................................… 13

- Theory of Pulse code modulation…………………………………...........14

4.1.Sampling  theorem …. ........................................................................……..14

4.2. Quantization process………….......................................................….…...15

4.3 Encoding and decoding techniques………………..............................…... 16

4.4 Bit rate calculation.....................................................................…….....….17

4.5. Signal recontruction…...................................................................……..….18

- IMPLEMENTION DETAILS……................................................…..21

5.1 Programming language and tools used…....................................…….23

5.2.Code structure and file organization..............................................…..……..23

5.3.Sampling module implementation......................................................….. 24

5.4Quantizer code logic………………..................................................….….25

5.5.Encoder and decoder code…………………..………………………..….26

5.6.Transmission simulate Module………............................................…....28

5.7.Audio signal handling…….............................................................….....29

  - Advantages and Disadvantages………….……………………………………….…..….31

RESULT ..................................................................................……..….33

FUTURE SCOPE..........................................................................................…..34

CONCLUSION............................................................................................…...35

REFERENCE…………………………………………………………………..41

# LIST OF FIGURES

Figure	Figure Name	PageNo

| 3.2 | PCM block diagram 8 |
| --- | --- |
| 5.4 | PCM output of decoder/encoder 32 |
| 5.6 | PCM output of audio signal 38 |

CHAPTER 1

### INTRODUCTION

#### Objectives of the project

The primary objective of this project is to study and implement the process of Pulse Code Modulation (PCM), a widely used technique in digital communication systems. PCM involves converting an analog signal into a digital format through three key steps: sampling, quantization, and encoding. This project aims to provide a practical understanding of how continuous-time analog signals are sampled at regular intervals, quantized into discrete levels, and then encoded into binary form for digital transmission. By simulating the PCM process, the project will demonstrate how digital signals can be transmitted over long distances with reduced noise and distortion compared to analog signals. The project also explores the importance of Nyquist’s theorem in determining the appropriate sampling rate to prevent aliasing. An additional goal is to reconstruct the original analog signal from the digital data and analyze the quality of the reconstructed signal. This helps in understanding the trade-offs between signal quality, bit rate, and bandwidth requirements. Through this implementation, the project highlights the significance of PCM in applications such as telephony, audio recording, and data storage. Overall, the objective is to gain both theoretical and hands-on knowledge of PCM and its role in modern communication systems.

#### 1.2 Scope of the project

The scope of this project encompasses the theoretical understanding, practical implementation, and performance analysis of Pulse Code Modulation (PCM) systems. It includes studying the fundamental concepts of analog-to-digital conversion such as sampling, quantization, and encoding. The project explores the importance of each step in ensuring accurate and efficient signal conversion. It also covers the role of sampling rate and bit depth in determining the quality and size of the digital signal. A major part of the scope involves simulating PCM using software tools or hardware kits to visualize how analog signals are transformed and transmitted. The project may extend to designing basic transmitter and receiver models for PCM communication. It evaluates the advantages of PCM, including noise immunity, easy signal regeneration, and compatibility with digital systems. Limitations such as quantization error, bandwidth requirements, and implementation complexity are also analyzed. The project scope includes comparing PCM with other modulation techniques like Delta Modulation. It also touches upon real-world applications in telecommunication, audio processing, and digital broadcasting. This

project can be further extended to study advanced encoding schemes or integration with compression techniques. Overall, the project offers a solid foundation for understanding digital communication systems and prepares the ground for exploring more complex modulation techniques.

  - SIGNIFICANCE OF PCM IN VOICE TRANSMISSION

Pulse Code Modulation (PCM) plays a vital role in voice transmission by enable the conversion of analog voice signals into digital format for efficient and reliable communication. In traditional telephony and modern VoIP systems, PCM ensures high-quality audio by accurately sampling the voice signal at regular intervals. With a standard sampling rate of 8 kHz and 8-bit quantization, PCM provides clear voice reproduction with minimal distortion. One of its key advantages is its resistance to noise and signal degradation during transmission, which is a common problem in analog systems. Since PCM works in the digital domain, it allows for error detection and correction, enhancing the integrity of the transmitted voice. It also supports multiplexing, enabling multiple voice signals to be transmitted simultaneously over a single channel. PCM is the foundation for TDM (Time Division Multiplexing) used in digital telephone networks. Moreover, digital signals can be compressed and encrypted easily, offering better security and bandwidth management. PCM is widely used in cellular networks, ISDN, and satellite communication. Its standardization has ensured global interoperability in telecommunication systems. Overall, PCM has transformed voice communication by making it clearer, more secure, and highly compatible with modern digital infrastructure.

#### OVER VIEW OF VOICE TRANSMISSION

Voice transmission is the process of transferring spoken sounds from one location to another through a communication system. It begins when a person speaks, creating sound waves in the air. These sound waves are captured by a microphone, which converts them into electrical signals, typically analog in nature. To enhance the quality and efficiency of transmission, these signals are often amplified, filtered, or compressed. In modern digital systems, the analog signals are further converted into digital data through a process called analog-to-digital conversion. Once processed, the signals are transmitted through various mediums such as copper wires, optical fibers, radio waves, or the internet. At the receiving end, the transmitted signals are decoded and converted back into sound waves by a speaker, allowing the listener to hear the original voice. Traditional landline telephones mainly used analog transmission, while today's mobile phones and internet-based communication platforms use digital transmission methods for faster and clearer voice delivery.

CHAPTER 2

### LITERATURE SURVEY

2.1   HISTORICAL  BACKGROUND OF PCM

Pulse Code Modulation (PCM) is a method for digitally representing analog signals and was first conceptualized by British engineer Alec Reeves in 1937. While working at International Telephone and Telegraph (ITT) in France, Reeves patented the idea of converting analog signals into a series of binary pulses, believing it would provide better noise resistance and improved communication quality. However, due to the limitations of electronic technology at the time, practical implementation was not immediately possible. During World War II, Reeves worked on secret military communication systems based on PCM principles, helping to demonstrate its value. In the 1950s and 1960s, advances in electronics, particularly the development of the transistor and improvements in digital processing, made PCM systems viable. Bell Labs played a major role in adapting PCM for telephone networks, enabling multiple voice signals to travel over the same lines with enhanced clarity and reliability. Today, PCM forms the foundation of many modern technologies, including audio CDs, digital telephony, internet communications, and high-definition audio systems.

#### 2.2   Existing   voice Transmission Technologies

Voice transmission technologies have evolved significantly, offering faster, clearer, and more reliable communication across the globe. Traditional methods like analog telephony, which transmitted voice signals directly over copper wires, have largely been replaced by digital systems. Modern technologies include Voice over Internet Protocol (VoIP), which converts voice into data packets and sends them over the internet, allowing for cost-effective and flexible communication. Cellular networks use digital transmission methods like GSM, CDMA, and LTE to support mobile voice calls with improved quality and coverage. Additionally, fiber-optic communication has enabled high-speed, high-capacity voice transmission over long distances with minimal signal loss. Satellite communication also plays a crucial role, especially in remote areas where traditional networks are unavailable. Together, these technologies ensure that voice communication is fast, clear, and accessible nearly everywhere in the world today.

#### 2.3   Comparison between Analog and digital Methods

#  ANALOG METHODS

.       Represents signals as continuous waves  (varying amplitude frequency, or phrase).

.     Closer to natural sounds or image forms

.      More  vulnerable to noise ,distortion, and signal loss over distance

.       Harder to compress, encrypt, or process compared to digital signals.

# DIGITAL METHODS

.       Represents signals as binary values (0s and 1s)

.        Highly resistant  to  noise and  distortion  maintaining quality over long distance

.        Easier to compress, encrypt . and intergrate with computer networks.

#### 2.4 GAPS IN EXISTING SYSTEMS

While Pulse Code Modulation (PCM) is widely used for reliable digital voice transmission, it still has certain gaps and limitations. PCM requires a relatively high bandwidth compared to more advanced compression methods, making it less efficient for bandwidth-limited channels. It also lacks built-in error correction, meaning that transmission errors can still affect the quality unless additional protocols are used. PCM systems are not very flexible with dynamic or variable data rates, limiting adaptability in modern, high-speed networks. Additionally, basic PCM does not inherently encrypt data, making security a concern in sensitive communications. Finally, in applications like mobile networks or internet streaming, newer encoding techniques such as Adaptive Differential PCM (ADPCM) or compressed formats like MP3 and Opus have largely replaced simple PCM to save bandwidth and improve efficiency.

PCM solid but struggle with  high bandwidth needs ,no built- in error correction low flexibility ,and security concerns

CHAPTER 3

SYSTEM DESIGN

### 3.1   PCM SYSTEM ARCHITECTURE   OVER VIEW

A PCM system mainly consists of three major stages:

- Sampling

  - The continuous analog signal is sampled at regular intervals.

  - According to the Nyquist theorem, the sampling rate must be at least twice the highest frequency present in the signal.

- Quantization

  - Each sampled value is approximated to the nearest value within a set of discrete levels.

  - This step introduces quantization error but enables digital representation.

- Encoding

  - The quantized values are then converted into a binary form (0s and 1s).

  - Each sample is represented by a fixed number of bits, creating a digital bit stream.

- Transmission

  - The binary-encoded signals are transmitted over a digital communication channel.

- Extra processes like framing, synchronization, and sometimes simple error detection may be added.d Reception and Reconstruction

  - At the receiver end, the binary data is decoded back into quantized levels.

  - A reconstruction filter (low-pass filter) smooths the signal to approximate the original analog waveform.

3.2   SYSTEM BLOCK DIAGRAM

#### 3.3 FUNCTIONAL COMPONENTS

1. Anti-aliasing Filter

- A low-pass filter.

- Removes high-frequency components before sampling to prevent aliasing.

2. Sampler

- Takes samples of the filtered analog signal at regular intervals.

- Sampling rate must satisfy the Nyquist criterion (at least twice the maximum signal frequency).

3. Quantizer

- Approximates (rounds off) the sampled values to a finite set of discrete levels.

- Introduces a small error called quantization noise.

4. Encoder

- Converts each quantized sample into a binary code.

- Produces the final digital PCM signal.

#### 3.4 DATA FLOW AND SIGNAL PATH

- Analog Input Signal →
(Continuous-time, continuous-amplitude signal.)

- Anti-Aliasing Filter →
(Smoothens and removes high-frequency noise from the input.)

- Sampler →
(Samples the filtered analog signal at regular time intervals.)

- Quantizer →
(Converts sampled continuous amplitudes into discrete amplitude levels.)

- Encoder →
(Encodes each quantized sample into a binary number.)

- Digital PCM Signal (Output) →
(A stream of binary numbers ready for transmission or storage.)

Analog Signal

Simple Signal Path Diagram

Analog Signal

↓

Anti-aliasing Filter

↓

sampler

↓

Quantizer

↓

Encoder

|

Digital PCM Signal

3.5     HARDWARE AND SOFTWARE REQUIREMENTS

*   HARDWARE REQUIREMENTS

  Analog-to-Digital Converter (ADC)

- For sampling, quantization, and encoding.

  Anti-Aliasing Filter (Low-Pass Filter)

- To filter the analog signal before sampling.

  Clock Generator

- To control the sampling rate precisely.

  Processor / DSP (Digital Signal Processor)

- For processing the quantized samples and encoding them efficiently.

  Memory / Storage Device

- To store the PCM data if needed.

  Transmission Medium (Optional)

- If sending PCM data (e.g., optical fiber, coaxial cable).

  Power Supply

.    For all active components

SOFTWARE  REQUIREMENTS

- Sampling Control Software

  - Programs that control timing and sampling rates.

- Quantization Algorithms

  - Software routines that assign discrete levels to samples.

- Encoding Algorithms

  - Codebooks or lookup tables for binary encoding.

- Error Detection and Correction (Optional)

  - Software to add redundancy or detect/correct transmission errors.

- Compression Software (Optional)

  - If PCM data needs to be compressed before transmission/storage.

- Data Management Software

  - For organizing, buffering, or streaming the PCM data.

CHAPTER 4

### THEORY OF PULSE CODE MODULATION

4.1 SAMPLING THEOREM

The Sampling Theorem, also known as the Nyquist Theorem, states that a continuous-time signal can be completely represented and perfectly reconstructed if it is sampled at a rate that is at least twice the highest frequency present in the signal. This minimum rate is called the Nyquist Rate. If the sampling frequency is less than twice the maximum signal frequency, a phenomenon called aliasing occurs, which causes distortion and loss of information. To prevent aliasing, an anti-aliasing filter is used before sampling to limit the signal's bandwidth. Thus, proper sampling is essential to accurately convert an analog signal into a digital form without losing important information.

If the highest frequency of the signal is fₘ,
then the minimum sampling rate fₛ must satisfy:

fs​≥2fm​

t

#### 4.2 QUANTIZATION PROCESS

Quantization is the process of mapping a continuous range of sampled signal values into a finite set of discrete levels. After a signal is sampled, the sample values can still take any value within a continuous range. To digitally represent these samples, each value must be approximated to the nearest predefined level. This rounding process introduces a small error known as quantization error or quantization noise. The number of quantization levels affects the quality of the signal — more levels result in higher accuracy and less distortion, while fewer levels lead to more noticeable errors but require less data to store or transmit. Quantization is a crucial step in Pulse Code Modulation (PCM), as it allows continuous analog signals to be converted into a digital format for efficient processing and communication.

#### 4.3 ENCODING AND DECODING

ENCODING

Encoding is the process of converting each quantized sample value into a unique binary code. After quantization, the sample values are represented by discrete levels, but to transmit or store them digitally, these levels must be expressed in binary form. Each quantized value is assigned a specific binary sequence based on its level number. The number of bits used for encoding depends on the number of quantization levels; for example, if there are 256 levels, 8 bits are required to represent each level. Encoding ensures that the digital representation of the signal is standardized and suitable for transmission, storage, and error checking. It is a critical step in Pulse Code Modulation, making the information fully digital and ready for further digital processing or communication.

DECODING

Decoding is the process of converting the received binary code back into its corresponding quantized signal value. In a PCM system, once the encoded digital data reaches the receiver, the decoder interprets each binary sequence and maps it to the original quantization level it represents. This process reconstructs a sequence of discrete amplitude values from the binary information. Although the decoded signal closely resembles the original sampled signal, it still contains some distortion due to quantization error. After decoding, additional processes like digital-to-analog conversion (DAC) and filtering may be applied to reconstruct a smooth analog waveform. Decoding is essential to recover the transmitted information and deliver it in a form that is usable for playback, display, or further analog processing.

#### 4.4  BIT RATE CALCULATION

The bit rate in PCM refers to the number of bits transmitted per second.
It depends on two main factors:

- Sampling rate (fₛ) — number of samples taken per second

- Number of bits per sample (n) — depends on the number of quantization levels

The formula is:

Bit Rate (R)=fs​×n

where:

- R = Bit Rate (in bits per second, bps)

- fs_= Sampling Frequency (samples per second)

- n= Number of bits per sample

#### 4.5 SIGNAL CONSTRUCTION

Signal Construction in Pulse Code Modulation (PCM) happens in these main steps:

- Sampling:

  - The analog signal is sampled at regular intervals.

  - Done according to the Nyquist theorem (sampling rate ≥ 2 × highest frequency in the signal).

  - Output: a sequence of sample values.

- Quantization:

  - Each sampled value is approximated to the nearest value from a finite set of levels.

  - This step introduces quantization error (small distortion).

- Encoding:

  - Each quantized value is then converted into a binary code (0s and 1s).

  - This binary data is what gets transmitted or stored.

Analog Signal → [Sampling] → [Quantization] → [Encoding] → PCM Signal (binary)

CHAPTER 5

### IMPLEMENTATION

Pulse Code Modulation (PCM) is implemented through a systematic process that converts an analog signal into a digital one. First, the analog input is sampled at regular intervals, a process governed by the Nyquist theorem to ensure that the sampling frequency is at least twice the highest frequency present in the signal. These discrete samples are then quantized, meaning each sample’s amplitude is rounded to the nearest value within a set of predefined levels, introducing a small quantization error. After quantization, each sample is encoded into a fixed-length binary code, typically using an encoder circuit or digital processor. The resulting stream of binary numbers can then be transmitted over digital communication channels or stored in memory. The bit rate of the PCM signal is determined by multiplying the sampling rate by the number of bits used per sample, which also influences the required bandwidth for transmission. This straightforward but precise process makes PCM a fundamental technique in modern telecommunications, audio recording, and data storage systems..

#### 5.1  PROGRAMMING LANGUAGE AND TOOLS

Programming Language: Python

In this PCM implementation, Python is chosen because it is highly suitable for signal processing simulations. Python provides an easy-to-write syntax, powerful libraries, and strong community support for digital signal processing (DSP) tasks. It allows fast prototyping and visualization, which are essential for experimenting with concepts like sampling, quantization, and reconstruction in PCM.

Tools and Libraries Used

| Library | Purpose |
| --- | --- |
| numpy | Efficient mathematical operations, array processing (normalization, quantization). |
| sounddevice | Capturing real-world audio from a microphone, playing back audio signals. |
| matplotlib | Plotting waveforms for visual analysis of original vs reconstructed signals. |

- numpy:

  - Used for normalization, quantization, and efficient signal transformations.

  - Functions like np.floor(), np.clip(), and array arithmetic allow handling signals quickly without explicit loops.

- sounddevice:

  - Allows recording of live audio from the microphone.

  - Functions sd.rec() and sd.play() help demonstrate PCM not just mathematically but also practically through hearing distortion introduced by quantization.

- matplotlib:

  - Helps visualize the difference between the original and PCM-reconstructed audio signals.

  - Useful for understanding effects like quantization noise and loss of signal fidelity.

#### 5.2 CODE STRUCTURE AND FILE AND ORGANIZATION

The entire Pulse Code Modulation (PCM) simulation is organized into one Python script file (example name: pcm_simulation.py).

Major Sections of the Code

| Section | Purpose |
| --- | --- |
| Imports | Load necessary libraries (numpy, sounddevice, matplotlib). |
| Global Settings | Define sample rate, duration of recording, and number of PCM bits. |
| Function Definitions | Separate reusable functions: pcm_encode(signal, bits), pcm_decode(pcm_data, bits). |
| Main Execution Block | Record audio, process PCM encoding/decoding, plot results, and optionally play back audio. |

a) Imports

- At the top, import all required libraries.

- No libraries are imported inside functions — everything is loaded globally for clarity and efficiency.

Import  numpy as np

Import  sounddevice as sd

import matplotlib.pyplot as plt

b) Global Settings

- Constants like sample_rate, duration, and bits are defined clearly in one place.

- Easy to modify later without touching the function code

sample_rate = 8000  # 8kHz typical for voice

duration = 2        # seconds

bits = 8            # 8-bit PCM

c) Function Definitions

- Functions are defined separately before being called.

- pcm_encode(signal, bits=8): Encodes a normalized signal into discrete PCM levels.

- pcm_decode(pcm_data, bits=8): Decodes PCM data back to approximate signal values.

- Keeps the code modular — easy to reuse, test, or improve without affecting other parts.

d) Main Execution Block

- Linear execution: record audio → encode → decode → plot → playback (optional).

- Audio is recorded once using sounddevice, and wait() ensures the recording is complete before processing.

- Time array is generated once for plotting both signals.

- matplotlib is used to plot two graphs (original and reconstructed) side by side for direct comparison.

#### 5.3 SAMPLING MODULE IMPLEMENTATION

In the given code, the primary aim of the sampling process is to convert the continuous-time analog audio signal from the microphone into a discrete-time digital signal suitable for further processing in Pulse Code Modulation (PCM). Sampling is performed by recording audio at a fixed rate of 8000 samples per second, ensuring that the signal captures enough information to accurately represent typical voice frequencies according to the Nyquist theorem. This process generates a sequence of amplitude values over time, effectively breaking the continuous signal into a series of individual data points. The sampled data, stored as a one-dimensional array, serves as the foundation for the subsequent quantization and encoding steps in the PCM simulation. Without accurate sampling, the integrity of the original analog signal would be lost, making it a critical first stage in the digital communication chain.

The sampling implementation in the code involves capturing real-time audio from the microphone and converting it into a series of discrete-time samples. This is achieved using the sounddevice library, which allows direct interaction with the system's audio input hardware. The sd.rec() function records the audio at a predefined sampling rate of 8000 Hz, meaning the sound is sampled 8000 times every second. The recording is done for a fixed duration of 2 seconds and in a single channel (mono) format, ensuring simplicity and clarity in the captured data. The recorded signal is initially stored as a two-dimensional array and then flattened into a one-dimensional array for easier processing. This discrete sequence of samples forms the digital representation of the original analog audio signal and serves as the input for the later stages of Pulse Code Modulation, such as quantization and encoding.

#### 5.4 QUANTIZER  CODE LOGIC

The quantizer maps a continuous signal (analog) into a discrete set of values, which is used in various digital signal processing applications like audio, image compression, etc.

Basic Quantizer Logic:

- Normalization: Normalize the input signal so that it falls within a fixed range (typically 0 to 1 or -1 to 1). This makes it easier to map the values to discrete levels.

- Quantization Levels: Decide the number of discrete levels to which the signal will be mapped. The number of levels is determined by the number of bits used for quantization. For example:

  - If you're using 8 bits, there will be 256 levels (2^8).

  - If you're using 4 bits, there will be 16 levels (2^4).

- Mapping: Each continuous value of the signal is mapped to the nearest discrete value (level). This is done by scaling the signal to the appropriate level and rounding it to the nearest quantization step.

- Scaling Back: After quantization, the signal is scaled back to the desired range.

  Quantization Levels: The number of distinct values that the signal can take. With n bits, the number of levels is 2^n.

  Quantization Error: The difference between the original signal and the quantized signal. This is also called quantization noise.

  Bit-depth: The number of bits used for quantization directly influences the number of levels and, consequently, the fidelity of the quantized signal. The higher the bit-depth, the more accurate the quantization.

#### 5.4  ENCODER AND DECODER CODE

In Pulse Code Modulation (PCM), the encoder transforms an analog signal into a digital form by first quantizing it into a set of discrete levels based on the number of bits specified (e.g., 8-bit, 16-bit). The process begins by normalizing the signal, mapping it to a defined range (usually between 0 and 1 or -1 and 1). The quantization step then maps the normalized signal to the nearest level, effectively reducing its continuous range to a finite number of discrete values. These quantized values are then encoded as digital data (e.g., binary or integer representations), ready for transmission or storage.

The decoder reverses this process, converting the digital signal back into an approximation of the original analog signal. The digital data is first scaled back to the original range (e.g., from [0, 1] to the signal's original amplitude range), and then dequantized, which reconstructs the continuous signal. However, due to quantization, some loss of fidelity (quantization error or noise) is inevitable, especially when the bit-depth is low.

Overall, PCM encoding and decoding allow us to represent analog signals in digital form, which is essential for applications like digital audio, telecommunication, and multimedia storage. The quality of the PCM signal depends on the bit-depth used for quantization—higher bit-depths provide more precise signal representation but require more data for storage and transmission.

import numpy as np

import sounddevice as sd

import matplotlib.pyplot as plt

from collections import deque

# Settings

sample_rate = 8000  # 8kHz

chunk_duration = 0.02  # 20ms

chunk_size = int(sample_rate * chunk_duration)  # Samples per chunk

bits = 8  # 8-bit PCM

plot_seconds = 2  # Plot last 2 seconds

# PCM functions

def pcm_encode(signal, bits=8):

signal_norm = (signal - signal.min()) / (signal.max() - signal.min())

levels = 2**bits

quantized = np.floor(signal_norm * (levels - 1))

return quantized.astype(np.uint8)

def pcm_decode(pcm_data, bits=8):

levels = 2**bits

signal_norm = pcm_data / (levels - 1)

return 2 * (signal_norm - 0.5)

# Buffer to store live data for plotting

buffer_size = int(sample_rate * plot_seconds)

audio_buffer = deque(maxlen=buffer_size)

recon_buffer = deque(maxlen=buffer_size)

# Real-time processing callback

def audio_callback(indata, frames, time, status):

if status:

print(status)

chunk = indata[:, 0]  # Take first channel if stereo

# Encode PCM

pcm_data = pcm_encode(chunk, bits)

# Decode PCM

reconstructed = pcm_decode(pcm_data, bits)

# Store for plotting

audio_buffer.extend(chunk)

recon_buffer.extend(reconstructed)

# Start stream

stream = sd.InputStream(samplerate=sample_rate, channels=1, callback=audio_callback, blocksize=chunk_size)

print("Starting real-time PCM stream... Press Ctrl+C to stop.")

try:

with stream:

fig, (ax1, ax2) = plt.subplots(2, 1, figsize=(12, 6))

while True:

if len(audio_buffer) > 0:

ax1.clear()

ax2.clear()

time_axis = np.linspace(-plot_seconds, 0, len(audio_buffer))

ax1.plot(time_axis, list(audio_buffer))

ax1.set_title("Original Audio (Live)")

ax1.set_xlabel("Time [s]")

ax1.set_ylabel("Amplitude")

ax1.grid()

ax2.plot(time_axis, list(recon_buffer), color='orange')

ax2.set_title(f"Reconstructed Audio after {bits}-bit PCM (Live)")

ax2.set_xlabel("Time [s]")

ax2.set_ylabel("Amplitude")

ax2.grid()

plt.pause(0.01)

except KeyboardInterrupt:

print("\nStopped streaming.")

import numpy as np

import sounddevice as sd

import matplotlib.pyplot as plt

from collections import deque

# Settings

sample_rate = 8000  # 8kHz

chunk_duration = 0.02  # 20ms

chunk_size = int(sample_rate * chunk_duration)  # Samples per chunk

bits = 8  # 8-bit PCM

plot_seconds = 2  # Plot last 2 seconds

# PCM functions

def pcm_encode(signal, bits=8):

signal_norm = (signal - signal.min()) / (signal.max() - signal.min())

levels = 2**bits

quantized = np.floor(signal_norm * (levels - 1))

return quantized.astype(np.uint8)

def pcm_decode(pcm_data, bits=8):

levels = 2**bits

signal_norm = pcm_data / (levels - 1)

return 2 * (signal_norm - 0.5)

# Buffer to store live data for plotting

buffer_size = int(sample_rate * plot_seconds)

audio_buffer = deque(maxlen=buffer_size)

recon_buffer = deque(maxlen=buffer_size)

# Real-time processing callback

def audio_callback(indata, frames, time, status):

if status:

print(status)

chunk = indata[:, 0]  # Take first channel if stereo

# Encode PCM

pcm_data = pcm_encode(chunk, bits)

# Decode PCM

reconstructed = pcm_decode(pcm_data, bits)

# Store for plotting

audio_buffer.extend(chunk)

recon_buffer.extend(reconstructed)

# Start stream

stream = sd.InputStream(samplerate=sample_rate, channels=1, callback=audio_callback, blocksize=chunk_size)

print("Starting real-time PCM stream... Press Ctrl+C to stop.")

try:

with stream:

fig, (ax1, ax2) = plt.subplots(2, 1, figsize=(12, 6))

while True:

if len(audio_buffer) > 0:

ax1.clear()

ax2.clear()

time_axis = np.linspace(-plot_seconds, 0, len(audio_buffer))

ax1.plot(time_axis, list(audio_buffer))

ax1.set_title("Original Audio (Live)")

ax1.set_xlabel("Time [s]")

ax1.set_ylabel("Amplitude")

ax1.grid()

ax2.plot(time_axis, list(recon_buffer), color='orange')

ax2.set_title(f"Reconstructed Audio after {bits}-bit PCM (Live)")

ax2.set_xlabel("Time [s]")

ax2.set_ylabel("Amplitude")

ax2.grid()

plt.pause(0.01)

except KeyboardInterrupt:

print("\nStopped streaming.")

Output of PCM ENCODER AND DECODER AUDIO SIGNAL

Top of Form

Bottom of Form

#### 5.6 TRANSMISSION SIMULATOR MODULE

A Transmission Simulator Module can simulate the process of transmitting a signal, either analog or digital, over a communication channel. This module typically includes signal encoding, noise introduction, and signal decoding to model how a signal would behave in a real-world communication system.

Here's an outline of how such a simulator could be built for a digital transmission system using PCM (Pulse Code Modulation) as the encoding scheme, along with noise introduction and signal recovery:

Steps in a Transmission Simulator:

- Signal Generation: Create or simulate a signal (analog or digital) to be transmitted.

- Encoding: The signal is encoded into a digital form using PCM, where it is quantized into discrete levels.

- Transmission Over Channel:

  - Introduce noise (e.g., Gaussian noise) to simulate real-world imperfections in transmission.

  - Optionally, include other transmission effects such as attenuation or delay.

- Receiving and Decoding: The received signal, now corrupted by noise, is decoded to recover the original signal as much as possible.

- Analysis: Measure the signal quality after transmission by comparing the received signal to the original one (e.g., through Signal-to-Noise Ratio (SNR) or Error Rate

#### 5.7 AUDIO SIGNAL HANDLING

An audio signal refers to a time-varying waveform that represents sound, typically captured in the form of analog or digital data. In its analog form, it is a continuous signal that varies in amplitude and frequency over time, corresponding to the fluctuations in air pressure caused by sound waves. When this sound is recorded, it is usually captured by a microphone, which converts the pressure variations in the air into electrical signals.

In digital audio, the analog audio signal is converted into a discrete set of samples through a process called sampling. Each sample represents the amplitude of the signal at a specific point in time. This process is known as analog-to-digital conversion (ADC). The sampling rate determines how often the audio signal is sampled per second, with common rates being 44.1kHz (CD quality) or 48kHz for professional audio. The bit depth determines the precision with which each sample is represented, with higher bit depths offering more detail and dynamic range in the audio signal.

Once digitized, the audio signal can be processed, stored, and transmitted in various ways, such as in audio files (e.g., WAV, MP3), audio streaming systems, or communications systems. Audio signals can be manipulated in many ways, including filtering, compression, and enhancement, to improve the quality or reduce the file size for storage and transmission. The quality of audio signals, whether analog or digital, plays a crucial role in various fields, such as music production, telecommunications, and sound engineering.

PROGRAMMING CODE  AUDIO TRANSMISSION

import numpy as np

import sounddevice as sd

import matplotlib.pyplot as plt

from collections import deque

# Settings

sample_rate = 8000  # 8kHz

chunk_duration = 0.02  # 20ms

chunk_size = int(sample_rate * chunk_duration)  # Samples per chunk

bits = 8  # 8-bit PCM

plot_seconds = 2  # Plot last 2 seconds

# PCM functions

def pcm_encode(signal, bits=8):

signal_norm = (signal - signal.min()) / (signal.max() - signal.min())

levels = 2**bits

quantized = np.floor(signal_norm * (levels - 1))

return quantized.astype(np.uint8)

def pcm_decode(pcm_data, bits=8):

levels = 2**bits

signal_norm = pcm_data / (levels - 1)

return 2 * (signal_norm - 0.5)

# Buffer to store live data for plotting

buffer_size = int(sample_rate * plot_seconds)

audio_buffer = deque(maxlen=buffer_size)

recon_buffer = deque(maxlen=buffer_size)

# Real-time processing callback

def audio_callback(indata, frames, time, status):

if status:

print(status)

chunk = indata[:, 0]  # Take first channel if stereo

# Encode PCM

pcm_data = pcm_encode(chunk, bits)

# Decode PCM

reconstructed = pcm_decode(pcm_data, bits)

# Store for plotting

audio_buffer.extend(chunk)

recon_buffer.extend(reconstructed)

# Start stream

stream = sd.InputStream(samplerate=sample_rate, channels=1, callback=audio_callback, blocksize=chunk_size)

print("Starting real-time PCM stream... Press Ctrl+C to stop.")

try:

with stream:

fig, (ax1, ax2) = plt.subplots(2, 1, figsize=(12, 6))

while True:

if len(audio_buffer) > 0:

ax1.clear()

ax2.clear()

time_axis = np.linspace(-plot_seconds, 0, len(audio_buffer))

ax1.plot(time_axis, list(audio_buffer))

ax1.set_title("Original Audio (Live)")

ax1.set_xlabel("Time [s]")

ax1.set_ylabel("Amplitude")

ax1.grid()

ax2.plot(time_axis, list(recon_buffer), color='orange')

ax2.set_title(f"Reconstructed Audio after {bits}-bit PCM (Live)")

ax2.set_xlabel("Time [s]")

ax2.set_ylabel("Amplitude")

ax2.grid()

plt.pause(0.01)

except KeyboardInterrupt:

print("\nStopped streaming.")

OUTPUT WAVE FORM OF ORIGINAL AUDIO SIGNAL

#### 5.8 ADVANTAGES AND DISADVANTAGES

Analog Audio Signals:

Analog audio refers to continuous signals that represent sound waves in their natural form. These signals are typically used in older audio equipment and tape recorders.

Advantages:

- High Fidelity: Analog signals can, in theory, represent the full range of sound frequencies and amplitudes without discrete steps. They are often praised for their natural, warm sound.

- No Quantization Noise: Unlike digital audio, analog signals don't suffer from quantization errors or "digital noise" that can degrade the sound quality, especially at low bit rates.

- Continuous Representation: Analog signals capture continuous changes in sound, which can be more accurate to the original sound source, as it’s not limited by sampling rates or bit depth.

Disadvantages:

- Susceptible to Noise: Analog signals degrade over time due to noise, distortion, and interference (e.g., from electrical components or environmental factors like humidity). This results in reduced sound quality.

- Limited Durability: Analog recordings (like on vinyl or magnetic tape) wear out over time, leading to loss of quality with each playback.

- Lack of Flexibility: Editing and processing analog audio signals are more challenging and less precise than digital systems. It’s also harder to store or manipulate large quantities of analog audio.

- Cumbersome Storage: Storing analog signals requires physical media (like tapes or vinyl), which are bulky and can be difficult to preserve over long periods.

Digital Audio Signals:

Digital audio represents sound using binary code, converting continuous analog signals into a series of discrete values through sampling and quantization.

Advantages:

- High Flexibility: Digital signals are easier to manipulate, edit, and process (e.g., mixing, effects, and filtering) without degradation in quality. Non-destructive editing allows for more creative control.

- Durability: Digital audio can be copied infinitely without degradation, making it ideal for storage and sharing. It’s less susceptible to physical damage compared to analog media.

- Efficient Storage: Digital audio can be compressed to save space (e.g., MP3, AAC), making it more efficient for storage and transmission. Large amounts of audio data can be stored and accessed quickly on digital media.

- Noise Resistance: Digital audio is less susceptible to noise and interference during transmission. Even if data corruption occurs, it can often be corrected or minimized without affecting the quality of the audio significantly.

- Precise Reproduction: With sufficient bit depth and sample rate, digital audio can offer very high quality and precise replication of the original sound. Modern digital formats can even exceed the limits of human hearing (e.g., 24-bit, 192kHz audio).

Disadvantages:

- Quantization Error: Digital audio suffers from quantization noise or aliasing at lower bit depths and sample rates. This can lead to a loss of subtle nuances, especially when the audio is compressed or when lower-quality settings are used.

- Compression Artifacts: While compression reduces file size, it may introduce artifacts such as popping, clicking, or distortion, particularly with lossy formats like MP3 or AAC.

- Requires Conversion: Analog signals must be converted into digital form (ADC), and digital signals must be converted back to analog (DAC) for playback through speakers. These conversions can introduce slight delays or errors if the equipment is not of high quality.

- File Size and Storage: Though digital audio can be compressed, high-quality, uncompressed formats (like WAV or FLAC) require significant storage space, especially for long or multi-track recordings.

- Sampling Limitations: If the sample rate or bit depth is not high enough, important parts of the audio (e.g., high frequencies or subtle dynamics) may be lost during the conversion to digital. Lower-quality audio formats can sound "harsh" or "unnatural" when compared to high-quality analog signals.

##### 5.9.1 Protection of data alteration

We take advantage of the fragility of the embedded data in this application area. The embedded data can rather be fragile than be very robust. Actually, embedded data are fragile in most steganography programs. However, this fragility opens a new direction toward an information alteration protective system such as a “Digital Certificate Document System.” The most novel point among others in that “no authentication bureau is needed.” If it is implemented, people can send their “digital certificate data” to any place in the world through internet. No one can forge, alter, nor tamper such certificate data. If forged, altered, or tampered, it is easily detected by the extraction program

##### 5.9.2 Access control system for digital content distribution

In this area embedded data is "hidden", but is "explained" to publicize the content. Today, digital contents are getting more and more commonly distributed by Internet than ever before. For example, music companies release new albums on their Webpage in a free or charged manner. However, in this case, all the contents are equally distributed to the people who accessed the page. So, an ordinary Web distribution scheme is not suited for a "case-by-case" and "selective" distribution. Ofcourse it is always possible to attach digital content to e-mail messages and send to the customers. But it will take a lot of cost in time and labor. If you have some valuable content, which you think it is okay to provide others if they really need it, and if it is possible to upload such content on the Web in some covert manner. And if you can issue a special "access key" to extract the content selectively, you will be very happy about it. A steganographic scheme can help realize this type of system. We have developed a prototype of an "Access Control System" for digital content distribution through Internet. The following steps explain the scheme.

- A content owner classify his/her digital contents in a folder-by-folder manner, and embed the whole folders in some large vessel according to a steganographic method using folder access keys, and upload the embedded vessel (stego-data) on his/her own Webpage.

- On that Webpage the owner explains the contents in depth and publicize worldwide. The contact information to the owner (post mail address, e-mail address, phone number, etc.) will be posted there.

- The owner may receive an access-request from a customer who watched that Webpage. In that case, the owner may (or may not) creates an access key and provide it to the customer (free or charged). In this mechanism the most important point is, a “selective extraction” is possible or not.

### CHAPTER 6 RESULTS

Pulse Code Modulation (PCM) is a highly effective method for transmitting voice signals in digital communication systems. By converting continuous analog voice signals into discrete digital form through sampling and quantization, PCM ensures high-quality voice transmission with minimal distortion. The technique offers several advantages, including noise resistance, as digital signals are less susceptible to interference compared to analog signals. PCM is widely used in modern telecommunication systems, such as VoIP and mobile networks, where it ensures clear and reliable voice communication. However, to optimize bandwidth usage and maintain transmission quality, compression techniques are often applied. While PCM offers excellent fidelity with higher sampling rates and bit depths, it can require significant bandwidth for uncompressed signals. In real-world applications, balancing quality, compression, and available bandwidth is essential to achieving efficient and high-fidelity voice transmission.

### CONCLUSION

Pulse Code Modulation (PCM) proves to be an efficient and reliable method for voice transmission in digital communication systems. Its ability to provide high-quality, noise-resistant voice signals makes it a key technology in modern telecommunication applications such as VoIP and mobile networks. While PCM offers excellent fidelity, it requires careful management of factors like sampling rate, bit depth, and compression to optimize bandwidth usage and maintain sound quality. Overall, PCM remains a cornerstone of digital voice transmission, providing clear and accurate communication even in challenging transmission environments.

### FUTURE SCOPE

The future scope of Pulse Code Modulation (PCM) in voice transmission is promising, with advancements in both technology and application areas. As demand for higher-quality, more efficient communication systems grows, PCM can evolve in several key areas:

- Higher Bit Depth and Sampling Rates: With the increasing availability of high-speed processors and storage, future systems may adopt even higher bit depths and sampling rates to provide superior audio fidelity. This would be especially beneficial for applications like professional audio recording, medical diagnostics, and high-definition voice calls.

- Advanced Compression Techniques: To optimize bandwidth usage without compromising on quality, advanced compression algorithms will likely evolve, reducing data usage while maintaining near-lossless audio quality. This is particularly important for streaming services, telecommunication networks, and cloud-based communication systems.

- 5G and Beyond: With the rollout of 5G networks and beyond, low-latency and high-bandwidth communication will make real-time voice transmission even more efficient. PCM could play a crucial role in ensuring high-quality, ultra-low-latency voice over these next-generation networks, opening up new possibilities for applications like virtual reality (VR) and augmented reality (AR) communication.

- Integration with AI and Speech Recognition: The future of PCM in voice transmission could see deeper integration with Artificial Intelligence (AI) and speech recognition technologies. AI could be used to enhance voice quality, reduce noise, and even provide real-time voice translation, making PCM-based systems more versatile in multilingual communication and automated voice assistance.

- Quantum Communication: As quantum computing and quantum communication technologies develop, they could introduce a new level of security and efficiency in PCM-based voice transmission, potentially revolutionizing how voice data is encrypted and transmitted.

- Wearable and IoT Devices: With the proliferation of wearable technology and IoT devices, PCM could be adapted to low-power, real-time voice transmission in small, efficient form factors. This could open up new opportunities in healthcare, smart homes, and wearable communication devices.

In summary, the future scope of PCM in voice transmission looks robust, with continuous improvements in fidelity, efficiency, and integration with cutting-edge technologies, ultimately leading to enhanced communication experiences across a variety of platforms and industries.

### REFERENCES

- Proakis, J. G., & Salehi, M. (2007). Digital Communications (5th ed.). McGraw-Hill.

  - This textbook provides a detailed explanation of digital communication systems, including PCM and its application in voice transmission.

- Mendel, J. M. (1997). Digital Signal Processing: A Computer-Based Approach. McGraw-Hill.

  - This book covers signal processing techniques, including PCM, and explains their implementation and use in various systems like telecommunications.

- Baker, R. (2008). Digital Signal Processing Handbook. CRC Press.

  - A comprehensive guide to the theory and application of signal processing, including PCM for audio and voice signals.

- Zhang, Z., & Dufresne, A. (2013). Voice over IP (VoIP): A Study of Voice Transmission Using Digital Communication Techniques. International Journal of Computer Applications, 75(2), 1-7.

  - This paper explores the use of PCM in Voice over IP (VoIP) systems, highlighting the impact of PCM on voice quality and transmission efficiency.

- Kuo, S. M., & Lee, W. (2007). Real-Time Digital Signal Processing: From MATLAB to C with the TMS320C6x DSPs. Wiley.

  - The book provides practical implementations of digital signal processing techniques such as PCM, focusing on real-time applications for voice and audio transmission.

- Schroeder, M. R. (2015). Introduction to Sound: Acoustics for the Hearing and Speech Sciences. Pearson.

  - This book gives a foundational understanding of acoustics and sound wave propagation, which is useful when understanding how PCM is used in voice signal transmission.

- Gustafson, P., & Johnson, T. (2006). PCM Voice Transmission in Modern Telecommunication Systems. IEEE Transactions on Communications, 54(4), 250-257.

  - This paper explores the real-world applications of PCM in telecommunications, including its strengths, limitations, and the future trends in voice transmission.

- Siddiqi, M. H. (2016). Voice and Speech Processing. Springer.

  - A resource that dives into voice and speech processing, with a particular focus on digital techniques such as PCM for voice transmission.

- ITU-T Recommendation G.711 (2008). Pulse Code Modulation (PCM) of Voice Frequencies. International Telecommunication Union.

  - This is the official standard by ITU for PCM and provides in-depth technical details on how PCM is implemented in telecommunication systems.

These references will provide you with both theoretical insights and practical knowledge related to PCM and its role in voice transmission systems.